Scope of Work

Project #5: North Dakota Pipeline

Client: Garrison Diversion Project and Red River Valley Water Supply Project

Course: CIVE 202 – Civil Engineering Analysis II

Team Members: Ayub Abdi, Eli Speth

Date submitted for review:

Submitted to:

Project Goals:

The main goal of the project is to determine whether pumping water from Lake Sakakawea through the McClusky Canal to the Sheyenne River will positively or negatively impact downstream water quality at Lake Ashtabula.

Some objectives in this project are:

- Determine if water quality worsens at the end of the McClusky Canal.

- Identify concerning water quality parameters and explain each one’s significance.

- Analyze the major anions, major cations, and other factors at each monitoring location.

- Create a model that estimates the total dissolved solids (TDS) concentrations in Lake Ashtabula.

- Develop a model that can be modified for separate scenario analysis.

This project will lead to a better understanding of the water supply in the eastern North Dakata region. It will address the problems that the McClusky Canal deals with.

Project Tasks:

- Preparing and Analyzing Data

- Import the USGS ND water quality data csv and format it for easier usage. Identify the main points of interest using the provided USGS location list.  Clean the dataset for the creation of graphs. Check all units to maintain consistency during calculations.

- Statistics

- For every monitoring location calculate the following:

mean, median, max, min.

- As well as the following parameters:

Major Cations: Calcium, Sodium, Magnesium, Sodium Absorption Ratio.

Major Anions: Chloride, Carbonate, Bicarbonate, Sulfate.

Other: Total Dissolved Solids, Temperature, pH, Hardness.

- Graph Analysis

- Create boxplots using matplotlib for four of the water quality parameters. Display different distributions by monitoring location along the McClusky Canal. Analyze the trends between Lake Sakakawea and the downstream canal locations.

- Mass Loading Calculation

- For each parameter use the mass loading calculation. Compare the differences between sites. ML = QxC. Q=(flow rate) and C=(concentration)

- Modeling

- Apply the mass balance equations to find the concentration at Lake Ashtabula. This model will show the flow rate of the pipeline and the Sheyenne Rover. It will also give the TDS concentrations.

- Water Quality Determination

- Compare upstream water quality to downstream water quality. Identify the parameters with the most change from typical thresholds. Evaluate any risks that are related to TDS, SAR, ecosystems, and drinking water.

- Provide Improvement Recommendations

- Provide any recommendations based on the data found. This could include treatment options, water relocation, or anything else necessary.

Project Deliverables:

- A .ipynb Jupyter Notebook file with Python code.

- Annotated Code Document that provides an explanation of each line of code.

- Cleaned csv dataset.

- Statistical Summary Tables, boxplots for four water quality parameters, mass loading calculations.

- TDS mixing model for Lake Ashtabula.

- Gantt and Flow Charts show the process in creating the code and completion of the project.

- Technical report including the results, recommendations, and any other discussion over the project.

- A GitHub repository with every file and a README file.

All deliverables will be uploaded to the GitHub repository and submitted by May 8th, 2026 at 5:00 PM CT.

Conclusion:

This project provides an analysis of the water quality from Lake Sakakawea to Lake Ashtabula. The water quality will be analyzed at many monitoring locations along the McClusky Canal and Sheyenne River. The water quality will show if the water being transported is degrading as it goes downstream. This will be done by evaluating the major actions, major cations, and other factors that can impact water quality. Boxplots, mass balance models, and stat summaries will be provided.

The mass balance model will allow for flow scenarios to be evaluated. This can help determine if any other action needs to be taken to prevent the degrading water quality. The results of this project will support any future water supply development in the eastern region of North Dakota.